Proposed final values for discrete components involving EE VFE readout, 29.11.2005, to be discussed

| Place | Old value/item | Proposed value/item | Comments |
| --- | --- | --- | --- |
| HV card blocking cap | 1nF | 475pF | Reduce energy of discharge to MGPA Agreed Oct 2005 |
| HV card, series resistor placed after blocking capacitor – resistor must be on HV card (MR) | Non existent | 100 | Slows down discharge Agreed to use, Oct 2005 |
| Motherboard | Non existent | 2 back to back diodes, Philips No longer??? | Copper gnd plane too thick for diodes?? |
| VFE card, C/R | 10pF/3.9k | 6.8pF/5.6 k | 12.8pC,~3.5ke |
| protection diodes | None | SOT882 package (3 pad, 35micron scale) | ASCOM can mount them. Availability questions |
|  | HV blocking capacitor on VFE card | zero ohm link |  |
|  | Surface mount components: |  | voltage ratings sufficient??? |
| HV card |  |  |  |
| Coax signal sheath to SC ground | 10M | May change if shown to be a noise source | Local ground connection |
| HV card unchanged |  |  |  |
| Dynode HV to R in series | 22M | No change |  |
| Capacitor to ground | 1nF | No change |  |
| - then to dynode itself |  |  |  |
| Anode HV to R in series | 10M | No change |  |
| Capacitor to ground | 1nF | No change |  |
| Anode HV to R in series | 10M | No change |  |
| - then to anode itself |  |  |  |
| Coax signal line resistor to sheath gnd | 10M | No change | Prevent signal line charge up |

http://www.hep.ph.ic.ac.uk/~dmray/pdffiles/VPT_umbilical_studies.pdf