##### EE Design Requirements

- Installation/Dismounting

It must be possible to mount/dismount a single Dee to HE with or without the beam pipe in position.

It must be possible to mount/dismount a single Dee with or without the second Dee in its final position.

- Maintenance of the Dees.

Following from 1) if one Dee has a problem it must be possible to mount/dismount the Dee without moving or uncabling the neighbouring Dee.

The guidelines imply that a Dee would first be loosened (slightly) and then moved horizontally away from the other Dee, along the x axis.

The Dee would then be brought forward (towards the IP) to clear the HE nose before being taken away by crane.