EE/HE FEA analysis.

- Abramov,1J. Greenhalgh2, A .Sourkov3

###### Mechanical analysis of EE

1. Geometry

- Drawings by J.Hill of backplate and flange.

- Crystal array – from report on EDMS, documents CMS-EC-ER-0001 to 0016, found at CMS/ECAL/ECAL Item classification/ECAL mechanics/ECAL endcap/EDR 2000 Endcap documents.

2. Supports

- Normal operation: all points around edge of ring flange are supported.

- Installation: supported by brackets.

3. Load cases

- Gravity in “y’ direction (normal operation and installation).

- Gravity in “x” direction (construction but there will be a brace).

- Seismic loads in x (3.5 m/s2), y (13.5m/s2), and z (3.5 m/s2).

4. Required output

- Max stress and deflection for all load cases, normal operation.

- Max stress and deflection for installation.

- Max loss of supercrystal clearance for all load cases, normal operation and for installation.

- Max loss of Dee clearance for z-wise seismic load.

5. Possible Variations

- Add bump stop and see effect on z-wise seismic results.

##### ANSYS analysis results of the CMS ECAL Endcap Backplate

# Operating mode.

| Acceleration (m/sec^2) | ### Super Crystals |  |  |  | ### Full Model |  | ### Back ### plate |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max. displacem. (mm) | Nearest to X=0; Ux (mm) | #### Clearance reduction (km) |  | Max. displacem (mm) | Max. von Mises (MPa) | Displacem. at X=0 (mm) |
|  |  |  | Horizontal | Vertical |  |  |  |
| Gravity: Gy=9.81 | 0.72 | -0.37 | 38.0 | 49.6 | 0.64 | 51 | -0.2 |
| Seismic: Gx=3.5, Gy=9.81 | 1.67 | -0.65 | 50.9 | 93.6 | 1.14 | 59 | -0.29 |
| Seismic: Gy=9.81, Gz=3.5 | 3.58 | -0.76 | 82.7 | 191.6 | 2.50 | 60 | -0.21 |
| Seismic: Gy=13.5 | 0.98 | -0.5 | 52.2 | 68.2 | 0.87 | 70 | -0.26 |
| Bump stop: Gy=9.81 | 0.70 | -0.37 | 38.1 | 48.1 | 0.64 | 51 | -0.2 |
| Bump stop: Gy=9.81, Gz=3.5 | 0.88 | -0.35 | 65.2 | 176.8 | 0.62 | 53 | -0.2 |

All displacements and clearance reductions for the SC were scaled by the factor 440/300. (Super Crystal length – 440mm and position pyramid apexes – 300mm from the backplate.)

##### ANSYS analysis results of the CMS ECAL Endcap Backplate

# Installation (with brackets)

| Acceleration (m/sec^2) | ### Super Crystals |  |  |  | ### Full Model |  | ### Back ### plate |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max. displacem. (mm) | Nearest to X=0 Ux (mm) | #### Clearance reduction (km) |  | Max. displacem (mm) | Max. von Mises (MPa) | Displacem. at X=0 (mm) |
|  |  |  | Horizontal | Vertical |  |  |  |
| Gravity: Gy=9.81 | 0.19 | -0.09 | 17.4 | 41.0 | 0.13 | 36 | -0.008 |
| Seismic: Gx=3.5, Gy=9.81 | 1.40 | -0.40 | 41.5 | 86.0 | 0.98 | 56 | -0.03 |
| Seismic: Gy=9.81, Gz=3.5 | 3.52 | -0.67 | 89.7 | 194.7 | 2.50 | 79 | -0.04 |
| Seismic: Gy=13.5 | 0.26 | -0.12 | 23.9 | 56.4 | 0.18 | 50 | -0.01 |
| Montage: Gx=9.81 | 3.40 | -0.87 | 108.0 | 161.4 | 2.4 | 71 | -0.07 |
| Bump stop: Gy=9.81, Gz=3.5 | 0.48 | -0.13 | 62.4 | 179.4 | 0.33 | 51 | -0.007 |

All displacements and clearance reductions for the SC were scaled by the factor 440/300. (Super Crystal length – 440mm and position pyramid apexes – 300mm from the backplate.)

|  | Material | Weight (kg) |
| --- | --- | --- |
| Backplate | aluminium | 826.0 |
| Flange (support ring) | aluminium | 191.0 |
| Super Crystals |  | 5964.0 (42kg-each) |
| Brackets | steel | 303.0 |

TOTAL  WEIGHT – 7282 kg.

1 abramov@mx.ihep.su

2 J.Greenhalgh@rl.ac.uk

3 Alexandre.Sourkov@cern.ch