Barrel Transport at Oxford – 9 June 2005

B4 arrives in Oxford from RAL

#1	“Oh no – the lorry’s empty … someone’s stolen B4 … Oh there’s the box, down the far end.”

#2	side of lorry – note no obstructions

#3	a hand was found in bottom of lorry – note surface of floor

#4	Mike Tyndel goes white with worry; removing B4

#5	forklift was brought on a separate lorry

#6	not much clearance

#7	note chocks under forks. Richard Nickerson did not think the forks reached to CoM and chocks were needed to wedge in forks – I don’t think this was quite true, but CoM would have been right over end of forks

#8	note box comes out horizontally

#9	box carried by forklift into loading bay – only few inches form ground

#10	forks with chocks on

#11	Shocklog on front of box

#12	front interior of lorry

#13	air-conditioning distribution in roof of lorry

#14	raising wheels before B4 extracted from outer box

#15	preparing platform for removal of inner box; front cover removed from outer box

#16	wheels on inner box

B6 leaves Oxford for CERN

#17	inside box before B6 inserted

#19	after attaching spring-plates, hoover up

#20	outer box awaits inner box at exit from clean room

#21	B6 emerging from clean room

#22	note cling-film (only around top ¾) and electrical connection between Bosch parts (Al-Al contact not good)

#23	note clips which fix panels onto steel uprights, rotated with Allen key on outside

#24	raising the inner box wheels in the outer box

#25	wheels fixed by cable ties to stop them rotating

#26	ready for outer panels

#27	moving B6

#28	Andy Park and his team load B6

#29	insert B6 into lorry

#31	Andy et al raise wheels in lorry; note one man is down at front (nor front door) to raise front wheels

#32	details of forklift

Other details:

Mike Tyndel prepared a nice letter for Customs which said “Don’t open ! If you need to look into box, contact Richard Nickerson”.

Means that Richard has to have his mobile switched on for duration of transport.

Lorry was 247 cm wide and 260 cm high.

Forks were 212 cm long.

Box was 360 cm long, with wheels protruding further.

Weight was 1.5-2.0 tonnes, with fairly even distribution of material (so CoM around 180 cm from end).